RCS Capstone Electrical Components

Pressure transducer

– spec sheet in Appendix A

Quantity = 1

Manufacturer Ashcroft

Part# G2 M01 15 F2 5000#

input voltage: 9-36Vdc

output voltage: 1-5Vdc

supply current 4: mA

Purpose

The pressure transducer will be used to measure remaining mass of N2 during flight for delta v calculations.

Time Sampling

Sample rate should be no less than 10 Hz

Data Transferred

The pressure transducer will output a voltage that needs to be interpreted as the pressure remaining in the tank

Connections

There are four wires inside a shielded cable.  Red and Black are for input voltage and White and grey for output.

System Location

The pressure transducer is located at the nozzle of the gas tank and has a 2 ft cable.

Solenoids

– spec sheet in Appendix B

Quantity: 6

Model# 320

Manufacturer: Humprey

Electrical Specs

Voltage: 24V DC w/ range of +10% and -15%

Resistance: 72 Ohms

Current: 333mA

Power Consumption: 8 W max draw

Response time: on - 0.018 s, off - 0.009 s

Purpose

The solenoids will be actuated to a given range of duty cycle by the rocket’s computer using a control algorithm.

Temperatures

Ambient temperature range: 32 F to 125 F

Connections

There are two 24 in black lead wires 18 AWG.

Humphry spec sheet on the 3rd page mentions two different connectors that can be ordered.

Appendix A – Pressure Transducer Data Sheet

Cable

Connection

Hirschmann

Connection

Metri-Pack

Connection

Flying Lead

Connection

Model G2 Pressure Transducer

APPLICATONS

The G2 pressure transducer combines

performance with value to meet the

demanding needs of the original equip-

ment manufacturer in applications

found in:

• Off-road Equipment

• Construction Machinery

• Performance Racing

• Railroad/Transportation

• Compressor Control

• HVAC and Refrigeration

• Agricultural Implements

• Process Automation and Control

• Hydraulic & Pneumatic Sensing

• Pump Monitoring

FEATURES

• 1% Total Error Band Accuracy

• Broad Temperature Capability

• All-welded pressure construction

• High EMI/RFI rating

• Ranges 30 psi through 20,000 psi

• IP 67 Ingress rating

• Diagnostic rails

PERFORMANCE SPECIFICATIONS

Ref. Condition 21°C ±1°C (72°F ±2°F)

Accuracy:

Total Error Band includes combined effects of temper-

ature, non-linearity (Terminal Point Method), hysteresis,

non repeatabilty, zero offset and span setting errors

±1% of Span: From –20 to 85ºC (–4 to 185ºF)

±1.5% of Span: From –40 to –20ºC (–40 to –4ºF)

±1.5% of Span: From 85 to 125ºC (185 to 257ºF)

Note: Static accuracy ±0.25% of span BFSL (Best Fit

Straight Line Method); includes non-linearity,

hysteresis and non-repeatable effects at reference

temperature 72°F (21°C)

Stability: Less than ±0.25% span/year

Durability: Tested to 50 million cycles

ENVIRONMENTAL SPECIFICATIONS

Temperature:

Compensated     –40 to 125°C (–40 to 257°F)

Operating            –40 to 125°C (–40 to 257°F)

Storage               –40 to 125°C (–40 to 257°F)

Humidity: 0 to 100% R.H., no effect

FUNCTIONAL SPECIFICATIONS

Select from over 25 pressure ranges starting at 30

psi and running through 20,000 psi gauge.

Compound (vacuum & pressure) ranges are also

available, see “To Order” on back.

Overpressure (F.S.):   Proof          Burst

750 psi & below     200% F.S. 1000% F.S.

1500 psi                  200% F.S. 500% F.S.

3000 psi                  200% F.S. 500% F.S.

5000 psi                  150% F.S. 500% F.S.

7500 psi                  120% F.S. 500% F.S.

10,000 psi               120% F.S. 240% F.S.

20,000 psi               120% F.S. 240% F.S.

Vibration: Random vibration (20 g) over tempera-

ture range (–40° to 125°C). Exceeds typical MIL.

STD. requirements

Shock: 100gs, 6 ms

Drop Test: Withstands 1 meter on concrete 3 axis

Response Time: Less than 1 msec

Warm-up Time: Less than 500 msec typical

Position Effect: Less than ±0.01% span, typical

ELECTRICAL SPECIFICATIONS

Output Signals Available:

Supply

Voltage Output             Excitation         Current

0-5 Vdc, 3 wire             9-36 Vdc         5mA

0-10 Vdc, 3 wire           14-36 Vdc         5mA

1-5 Vdc, 3 wire             9-36 Vdc         4mA

1-6 Vdc, 3 wire             9-36 Vdc         4mA

0.5-4.5Vdc, 3 wire        9-36Vdc          4mA

Ratiometric Output:

0.5-4.5 Vdc, 3 wire       5 Vdc ±0.5 Vdc 3.5mA

Current Output:

4-20mA, 2 wire            9-36 Vdc

Reverse Polarity & Miswired Protected: Yes

Insulation Breakdown Voltage: 100 Vac

Insulation Resistance: Greater than 100 megohms

at 100 Vdc

CE Marked: Per DoC

EMC Directive 2004/108/EC

IEC/EN 61326-1:Edition 1.0 Industrial

IEC/EN 61326-2-3:Edition 1.0 Annex BB Industrial

TOTAL ERROR BAND (TEB)

Error limits of all points (0-100% of range)

Ambient Temperature (C)

-60	-40	-20 	0	20	40	60	80	100	120	140

2.0%

1.5%

1.0%

0.5%

0.0%

-0.5%

-1.0%

-1.5%

-2.0%

Pos Error Limit

Neg Error Limit

The Ashcroft® Type G2† pressure transducer

has been specifically designed with the high

volume OEM in mind.

A ±1% Total Error Band accuracy is accom-

plished by marrying a high performance ASIC

to a very stable, field proven polysilicon thin

film pressure sensor. The sensor is electron

beam welded to a pressure fitting of stainless

steel, which  provides excellent overpressure

capability and outstanding durability in the

presence of shock and vibration.

The circuitry is held within an internal cage

and housed in an enclosure of reinforced Nylon.

LOOK FOR THIS

AGENCY MARK ON

OUR PRODUCTS

PED Directive

BULLETIN G2

Ashcroft Inc., 250 East Main Street, Stratford, CT 06614 USA

Tel: 203-385-0648 • Fax: 203-385-0408

email: info@ashcroft.com • www.ashcroft.com

All specifications are subject to change without notice.

All sales subject to standard terms and conditions.

© 2013 Ashcroft Inc., Rev. 07/14

10	40

9 (min)	36 (max)

Loop Supply Voltage (Vdc)

[LSV]

Power Supply Voltage vs. Loop Resistance

(4-20mA ONLY)

1400

1200

Model G2 Pressure Transducer

BULLETIN G2

How To Order

G  2	7           	X

Type

Configuration

(G2)

Pressure Connection

Electrical Termination

Accuracy

1.0% Total Error Band

–20°C/+85°C

1.5% Total Error Band

–40°C/-20°C, 85/125°C

05  = 0-5 Vdc

10  = 0-10 Vdc

15  = 1-5 Vdc

16  = 1-6 Vdc

42  = 4-20mA

45  = 0.5-4.5Vdc

9-36Vdc supply

RM= 0.5-4.5 Vdc

Ratio Metric to

5Vdc supply

Consult Factory for

Available Options

Pressure Ranges	Optional

X-Variations

M011⁄8NPT-male

M02 1⁄4NPT-male

MEK 7⁄16-20 SAE-male

w/Buna-N O-ring

M38 3⁄8-24 SAE-male

w/Buna-N O-ring

MEV9⁄16-18 SAE-male

w/Buna-N O-ring

M333⁄8-24 UNJF3A (w/37° cone seat)

M76 7⁄16-20 UNJF-3A (w/37° cone seat)

MS21⁄4-19 bsp male

MG2G 1⁄4B male

Consult factory for other connections

psi Ranges

30#= 30 psi

50#= 50 psi

60#= 60 psi

100#= 100 psi

150#= 150 psi

200#= 200 psi

300#= 300 psi

400#= 400 psi

500#= 500 psi

750#= 750 psi

1000#= 1000 psi

1500#= 1500 psi

2000#= 2000 psi

3000#= 3000 psi

4000#= 4000 psi

5000#= 5000 psi

6000#= 6000 psi

7500#= 7500 psi

10000#= 10000 psi

15000#= 15000 psi

20000#= 20000 psi

Metri-Pack*

GN = no mating conn.

G2  = mating conn. 3´ cable

G3  = mating conn. 10´ cable

G1  = mating conn. w/customer

specified length

Hirschmann G Series**

HM = no mating conn.

M1 = with mating conn. no cable

M2 = mating conn. 3´cable

P9  = mating conn. w/customer

specified length

Flying Leads

W2 = 3´flying leads

W9 = customer specified length

Shielded Cable

F2  = 3´shielded cable

F3  = 10´shielded cable

P1  = customer specified length

M12 –Mates to Hirschmann

933 172-100 or similar

EW = w/o mating conn.

E0  = w/mating conn. no cable

E2  = w/mating conn. &

3´shielded cable

E1  = w/mating conn. &

Deutsch DT Series DT04-3P

DT  = w/out mating conn.

T2  = w/1m, 3´cable

T3  = w/3m, 10´cable

T1  = w/mating conn. cable

customer defined length

Deutsch DTM Series DTM04-3P

DS  = w/out mating conn.

S2  = w/1m, 3´cable

S3  = w/3m, 10´cable

S1  = w/mating conn. cable

customer defined length

AMP Superseal

AP  = w/out mating conn.

A2  = w/1m, 3´cable

A3  = w/3m, 10´cable

A1  = w/mating conn. cable

customer defined length

EN 175301-803, Form C

(DIN 43650, Form C)

DC	= no mating conn.

N1	= with mating conn. no cable

N2	= mating conn. 3´cable

N3	= mating conn. 10´cable

N9	= mating conn. w/customer

specified length

Output Signal	Measurement Type

G= Gauge Pressure

VdcMIN = 9V + [0.022AX (RL]

RL= RS+ RW

RL= Loop Resistance (ohms)

RS= Sense Resistance (ohms)

RW= Wiring Resistance (ohms)

Compound Ranges

30#&vac= 30 psi/-14.7 psi

45#&vac= 45 psi/-14.7 psi

60#&vac= 60 psi/-14.7 psi

85#&vac= 85 psi/-14.7 psi

100#&vac= 100 psi/-14.7 psi

150#&vac= 150 psi/-14.7 psi

200#&vac= 200 psi/-14.7 psi

300#&vac= 300 psi/-14.7 psi

PHYSICAL SPECIFICATIONS

Wetted Materials: 304SS pressure connection and

17-4PH SS sensor diaphragm

Housing: 20% Glass Reinforced Nylon,

Fire retardant to UL94 V1

Available Process Connections (Male):

See “How To Order” section below. For other

connections consult factory.

Ingress Rating:

• IP67, NEMA 4X:

- Metri-Pack I50 series*

- Shielded cable

- Flying leads

- Deutsch DT Series DT04-3P

- Deutsch DTM Series DTM04-3P

- AMP Superseal

- M12 - Mates to Hirschmann 933 172-100 or similar

• IP65, NEMA 4X:

- Hirschmann G series**

- EN 175301-803, Form C (DIN 43650, Form C)

ELECTRICAL TERMINATION

See “How To Order” section below for electrical

termination options.

DIMENSIONS

customer defined length

Ashcroft Inc., 250 East Main Street, Stratford, CT 06614 USA

Tel: 203-385-0648 • Fax: 203-385-0408

email: info@ashcroft.com • www.ashcroft.com

All specifications are subject to change without notice.

All sales subject to standard terms and conditions.

© 2013 Ashcroft Inc., Rev. 07/14

1.40

1.59

.23

.94

1.06

(27)

1.06

(27)

(36)

MINI HIRSCHMANN**

(24)

.94

(24)

.71

(18) 2.24

(57)

(6)

2.05

(52)

.30 (8)

.23

(6)

(40)

Wire

Length

AMP SUPERSEAL	DEUTSCH DT04-3P	DEUTSCH DTM04-3P	3-PIN METRI-PACK*

.23

Ø.94	2.03

.45

1.02

1.06	.54

2.11

.56

.23

.94

.95

1.13

1.06

.44

.23

2.04

.46

Ø.94

1.06

.81

.67

Optional

Mating

Connector

Optional

Mating

Connector

Mates to Metri-Pack

Connector Body 12065286

Mates to Hirschmann G4W1F

Connector or Similar

EN 175301-803, Form C

(DIN 43650, Form C)

M12 4-POLE PLUG

Ø0.95

[Ø24.0]

1.69

[42.9]

1.74

[44.2]

2.60

[65.9]

0.23

[5.7]

1/4 NPT

Connection - PIN

Vs+

1

V out	3	2

Vs-

3.992 TYP

Install height

Pin 1

Pin 3	Pin 2

Pin 4

**Metri-Pack is a trademark of Delphi Packard Electric Systems.

**Trademark of Richard Hirschmann of America, Inc.

Appendix B – Solenoid Data Sheet